Texas A&M mini MPL Operation SOP

Operator Name: ____________	Operation Date: ____________

- Start lidar measurement

  - Connect the lidar power cables and data cables 					□

  - Turn on the lidar power rocker switch						□

  - Unscrew the laser cover								□

  - Turn on the key and the red rocker switch to turn on the laser			□

  - Do not turn off laser (Both key and switch) while the MPL software is collecting data.

  - Open sigmaMPL software if not automatically opened, open File -> Real time hardware control									□

  - Software may automatically start collecting. Check config files and resolution, stop if need to change.									□

  - In configuration, record Lidar Afterpulse file:  ____________________________________

  - Record Lidar Overlap file:   _____________________________________________________

  - Record Deadtime file: _________________________________________________________

  - Record Lidar temporal resolution: _______ and vertical resolution: _______

  - Click Save Data, make sure the entry box on the right side is empty		□

  - Click “Start Collecting”. Record Lidar Laptop Time:  ______________________________

  - Wait for the first green bar to finish, and record the file name:   ____________________

- During lidar measurement

  - Monitor Raw, NRB, Depol, and Hardware tab and record field notes:

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

  - If the laser temperature reaches 35 °C, stop collecting and turn off the laser

- Shut down procedure

  - Click stop collection									□

  - Close software									□

  - Turn off the laser (key and then the red rocker switch). Make sure laser is off. 	□

  - Flip rocker switch off									□

  - Screw the cover back in and unplug cables.					□